Copyright    Massachusetts Institute of Technology    1985

This note describes version ten of window system X.

The X protocol can be built on top of any reliable byte stream.  A simple block

protocol is used on top of the stream layer.  Most requests are 24 byte blocks;

some carry additional data.  Requests do not have a corresponding reply unless

data is actually expected in return.  This means that error responses in

general will not be caught at the failing request, but rather at some later

request which expects a reply.

On a given machine, displays are numbered from zero.

For TCP connections, display N is associated with port 5800+N ('X' = 0x58) and

with port 5900+N.  That is, the X server listens and accepts new connections on

these ports.  The X server expects connections made on the 58xx port to come

from hosts transmitting data in little Indian (VAX order), and expects

connections made on the 59xx port to come from hosts transmitting data in big

Indian (68000 order).  For connections with opposite-order machines, the server

will byte swap all 16-bit and 32-bit quantities in all requests and replies,

with the exception of host addresses, which should be transmitted in standard

network order.  In this way, clients need never byte-swap, and byte-swapping

need not occur between hosts with the same byte ordering.

For Unix-domain connections, display number ? associated with path /dev/X?.

For DECnet connections, display ? is associated with object name "X?".

The coordinate system has X horizontal and Y vertical, and is in terms of

pixels.  The point (0, 0) is the upper left.  Coordinates within a window are

always specified in terms of the inside of a window (not including the

borders).  A pixel is an N-bit value, where N is the number of bit planes

provided by the particular display; a pixel value indexes a color map to derive

an actual color to be displayed.

Some C type definitions used below:

typedef long Window;

typedef long Font;

typedef long Bitmap;

typedef long Pixmap;

typedef long Cursor;

typedef long Locator;

For the convenience of certain programming languages, Window, Font, Bitmap,

Pixmap, and Cursor values are guaranteed to have the top three bits all zero.

A Bitmap is a single plane (bit) rectangle.  A Pixmap is an N-plane (pixel)

rectangle, where N is the number of planes provided by the particular display.

In this protocol, N ranges from 1 to 16.  Cursors are used as mouse pointers; a

cursor is an arbitrary two-color shape with an arbitrary "point".  A Locator is

an absolute point on the display, represented as <x,,y> with the X-coordinate

in the high 16 bits and the Y-coordinate in the low 16 bits.

A Bitmap is represented by ((width + 15) / 16) * height * 2 bytes of data.  The

bits are in scanline order, with each scanline padded if necessary to a

multiple of 16 bits.  The pad bits are of arbitrary value.  Within a scanline,

the bits are represented left to right, stored in 16-bit words, but the bits

are reversed within a given 16-bit word: the leftmost bit of the scanline is

the least significant bit of the word.

A Pixmap can be represented in either "XY-format" or "Z-format".  In XY-format,

each plane is represented as a Bitmap, and the planes appear from most to least

significant bit order.  The total number of bytes is thus

((width + 15) / 16) * height * 2 * depth.  In Z-format, the pixels are in

scanline order, left to right within a scanline.  For displays with 2 to 8

planes, each pixel is represented by a single byte; the total number of bytes

is thus width * height.  For displays with 9 to 16 planes, each pixel is

represented by a 16-bit word; the total number of bytes is thus

2 * width * height.  Z-format cannot be used on monochrome displays.

Pixel values 0 and 1 are always defined on every display.  They are primarily

intended for use in monochrome applications.  On monochrome displays, pixel

value 0 is black and pixel value 1 is white.  On color displays, the colors may

be redefined by the user.

Display function codes used in all output requests, and their affect on the

destination as a function of the source and the destination (NOT binds

tightest) are:

GXclear			0x0		0

GXand			0x1		src AND dst

GXandReverse		0x2		src AND NOT dst

GXcopy			0x3		src

GXandInverted		0x4		NOT src AND dst

GXnoop			0x5		dst

GXxor			0x6		src XOR dst

GXor			0x7		src OR dst

GXnor			0x8		NOT src AND NOT dst

GXequiv			0x9		NOT src XOR dst

GXinvert		0xa		NOT dst

GXorReverse		0xb		src OR NOT dst

GXcopyInverted		0xc		NOT src

GXorInverted		0xd		NOT src OR dst

GXnand			0xe		NOT src OR NOT dst

GXset			0xf		1

Given a source and destination pixel, a display function is computed bitwise on

corresponding bits of the pixels.  That is, a boolean operation is performed on

each bit plane of the display.  A "plane mask" can be used to restrict output

operations to a subset of planes; the mask contains a one bit for each pixel

bit (plane) that should be affected.

The C definition of a request looks like:

typedef struct _XReq {

unsigned char code;

unsigned char func;

unsigned short mask;

Window windowId;

union {

long l[4];

short s[8];

unsigned short u[8];

char b[16];

} param;

} XReq;

Note: requests carrying more than 128K bytes of additional data will cause the

server to close the client's connection.

Input events (keyboard, mouse button, mouse motion, window change) are

generated asynchronously by X and are reported on the same network connection.

Clients must therefore expect any number of input events between any two

replies to requests.  Data coming from X is bundled into 24 byte blocks, with a

code indicating the type of data.  The C definition of this structure is:

typedef struct _XRep {

long code;

union {

long l[5];

short s[10];

unsigned short u[10];

char b[20];

} param;

} XRep;

The possible XRep codes are:

X_Reply		0		Normal reply

X_Error		-1		Error

else				Asynchronous input event

The contents of event structures are discussed later.  The contents of X_Reply

structures vary with each request, and are documented for each such request.

The contents of an X_Error structure is as follows:

code		X_Error

param.l[0]	number of failing request

param.b[4]	error code

param.b[5]	original request code

param.b[6]	original request func

param.l[2]	original request windowId

Requests are counted per network connection, starting from one.  The possible

error codes are:

BadRequest	1		bad request code

BadValue	2		int parameter out of range

BadWindow	3		parameter not a Window

BadPixmap	4		parameter not a Pixmap

BadBitmap	5		parameter not a Bitmap

BadCursor	6		parameter not a Cursor

BadFont		7		parameter not a Font

BadMatch	8		parameter mismatch

BadTile		9		Pixmap shape invalid for tiling

BadGrab		10		mouse/button already grabbed

BadAccess	11		access control violation

BadAlloc	12		insufficient resources

BadColor	13		no such color

Not all displays support all variations of all output requests.  Output

requests may be transformed arbitrarily (including being ignored), without

errors being generated.

Some displays have very limited memory for storage of off-screen resources such

as bitmaps, pixmaps, and fonts.

All requests and replies are padded as necessary to be a multiple of four bytes

long.  The pad bytes are of arbitrary value.

The requests are:

X_CreateWindow

code		1

func		border width

windowId	parent Window

param.s[0]	inside height (not including borders) (> 0)

param.s[1]	inside width (not including borders) (> 0)

param.s[2]	outer left coord (start of border)

param.s[3]	outer top coord (start of border)

param.l[2]	border tile Pixmap or 0

param.l[3]	background tile Pixmap or 0

Creates an unmapped (not displayed) opaque window.  Coordinates are

relative to the inside of the parent.  A border pixmap need not be

given if the border width is zero.  If no background pixmap is given,

the parent's background pixmap is used.  The tilemode of the new window

is TileModeAbsolute.  The clipmode of the new window is

ClipModeClipped.  The window does not have a defined cursor.

The parent window must be an opaque window.

A window will always be wholly contained within its parent, i.e. any

parts of the window that extend outside the parent window will not be

displayed.

The background and border pixmaps may be freed immediately if no

further explicit references to them are to be made.

The reply:

param.l[0]	Window

Errors: BadWindow, BadValue, BadPixmap, BadTile, BadMatch

X_CreateTransparency

code		2

windowId	parent Window

param.s[0]	inside height (> 0)

param.s[1]	inside width (> 0)

param.s[2]	outer left coord

param.s[3]	outer top coord

Creates an unmapped (not displayed) transparent window.  Coordinates

are relative to the inside of the parent.  The tilemode of the new

window is TileModeRelative.  The clipmode of the new window is

ClipModeClipped.  The window does not have a defined cursor.

The reply:

param.l[0]	Window

Errors: BadWindow, BadValue

X_DestroyWindow

code		3

windowId	Window

Unmaps and destroys the window and all of its subwindows.  The windows

should never again be referenced.

Windows are automatically destroyed when the creating process closes

its network connection.

If the window has a mapped icon window, that icon window is unmapped

and receives an UnmapWindow event.  If the window is a mapped icon

window, its corresponding "regular" window is mapped.

Generates exposure events on formerly obscured opaque windows.

Errors: BadWindow

X_DestroySubwindows

code		4

windowId	Window

Destroy all subwindows of this window.  The windows should never again

be referenced.

Generates exposure events on formerly obscured opaque windows.

Errors: BadWindow

X_MapWindow

code		5

windowId	Window

Maps and raises the window and displays the window and all of its

subwindows which have had map requests.  The previous contents of all

opaque windows are lost; mapping transparent windows does not affect

the screen.

Mapping a window when one of its ancestors is unmapped does not cause

the window to be displayed.

Has no effect if the window is already mapped.

Generates an ExposeWindow event on each newly displayed opaque window.

Errors: BadWindow

X_MapSubwindows

code		6

windowId	Window

Maps all subwindows of the given window in an unspecified order.

Generates an ExposeWindow event on each newly displayed opaque window.

Errors: BadWindow

X_UnmapWindow

code		7

windowId	Window

Unmaps the window and all of its subwindows.  Unmapping transparent

windows does not affect the screen.

Generates an UnMapWindow event on the window if it is mapped; generates

exposure events on formerly obscured opaque windows.

Errors: BadWindow

X_UnmapSubwindows

code		8

windowId	Window

Unmaps all subwindows of the given window.

Generates an UnMapWindow event on each mapped subwindow; generates

exposure events on formerly obscured opaque windows.

X_UnmapTransparent

code		9

windowId	Window

Unmaps the window and all of its subwindows, but does not affect the

screen, even if the window is opaque, and does not generate any

exposure (or unmap) events.

Errors: BadWindow

X_RaiseWindow

code		10

windowId	Window

Raise this window above all sibling windows, so that no sibling

obscures it.

Raising a transparent window does not affect the screen.  Transparent

windows never obscure other windows for the purposes of output, but do

obscure for the purposes of cursor and input control.

Window hierarchies never interleave.  If window A is obscured by window

B, then window A will only obscure ancestors of B that are also

ancestors of A.

Generates exposure events on formerly obscured opaque windows.

Errors: BadWindow

X_LowerWindow

code		11

windowId	Window

Lower this window below all sibling windows, so that it doesn't obscure

any siblings.

Raising a transparent window does not affect the screen.

Generates exposure events on formerly obscured opaque windows.

Errors: BadWindow

X_CircWindowUp

code		12

windowId	Window

Raise the lowest mapped child of this window that is (partially)

obscured by another child.  Repeated executions lead to round robin

raising.

Generates exposure events on formerly obscured opaque windows.

Errors: BadWindow

X_MoveWindow

code		13

func		0 (for future expansion)

windowId	Window

param.s[0]	outer left coord (start of border)

param.s[1]	outer top coord (start of border)

Moves and raises the window, without changing its size.  Coordinates

are relative to the inside of the parent.

The contents of an opaque window are lost if its tilemode is relative

or if the window is obscured by non-children.  Moving a transparent

window does not affect the screen.

Generates an ExposeWindow event on the (opaque) window if its contents

are lost; generates exposure events on formerly obscured opaque

windows.

Errors: BadWindow

X_ChangeWindow

code		14

func		0 (for future expansion)

windowId	Window

param.s[0]	inside height (not including borders) (> 0)

param.s[1]	inside width (not including borders) (> 0)

Changes the size of the window and raises it, without changing its

upper left hand coordinate.  The contents of an opaque window are lost;

changing a transparent window does not affect the screen.

Generates an ExposeWindow event on the (opaque) window; generates

exposure events on formerly obscured opaque windows.

Errors: BadWindow, BadValue

X_ConfigureWindow

code		15

func		0 (for future expansion)

windowId	Window

param.s[0]	inside height (not including borders) (> 0)

param.s[1]	inside width (not including borders) (> 0)

param.s[2]	outer left coord (start of border)

param.s[3]	outer top coord (start of border)

Changes the size and placement of the window and raises it.  The

contents of an opaque window are lost; configuring a transparent window

does not affect the screen.

Generates an ExposeWindow event on the (opaque) window; generates

exposure events on formerly obscured opaque windows.

Errors: BadWindow, BadValue

X_ChangeBackground

code		16

windowId	Window

param.l[0]	background tile Pixmap or 0

Change the background tile of a window.  If no background pixmap is

specified, the background pixmap of the window's parent is used (on

the root window the default background is restored).

Does not change the current contents of the window.

Can only be performed on an opaque window.

The pixmap can be freed immediately if no further explicit references

to it are to be made.

Errors: BadWindow, BadMatch, BadPixmap, BadTile

X_ChangeBorder

code		17

windowId	Window

param.l[0]	border tile Pixmap

Change the border tile of a window and repaint the border.

Can only be performed on an opaque window that has a border.

The pixmap can be freed immediately if no further explicit references

to it are to be made.

Errors: BadWindow, BadMatch, BadPixmap, BadTile

X_TileMode

code		18

func		0: TileModeAbsolute

1: TileModeRelative

windowId	Window

Sets the tilemode of the window.  With TileModeAbsolute (the normal

case for opaque windows), tiles are laid out with the upper left corner

of the window as an effective origin.  With TileModeRelative (the

normal case for transparent windows), tiles are laid out with the upper

left corner of the closest parent window with an absolute tilemode as

an effective origin.  The tilemode affects painting of the background

for exposures and for X_Clear, as well as the X_TileFill and

X_DrawFilled requests.

Does not change the current contents of the window.

Errors: BadWindow, BadValue

X_ClipMode

code		19

func		0: ClipModeClipped

1: ClipModeDrawThru

windowId	Window

Sets the clipmode of the window.  With ClipModeClipped (the normal

case), future output to the window will be obscured by subwindows.

With ClipModeDrawThru, future output to the window will ignore

subwindows and draw into them.  In draw-thru mode, the most useful

display functions are GXxor and GXinvert, so that displaying again will

erase what was displayed.  Draw-thru mode is useful for drawing window

outlines when moving or resizing windows.

The root window starts with ClipModeDrawThru.

Errors: BadWindow, BadValue

X_QueryWindow

code		20

windowId	Window

Get facts about the window.

The reply:

param.s[0]	inside height (not including borders)

param.s[1]	inside width (not including borders)

param.s[2]	outer left (start of border)

param.s[3]	outer right (start of border)

param.s[4]	border width

param.b[10]	0: IsUnmapped

1: IsMapped

2: IsInvisible (mapped but some ancestor is unmapped)

param.b[11]	0: IsTransparent

1: IsOpaque

2: IsIcon

param.l[3]	icon Window or opaque Window or 0

If windowId is a transparent window, param.b[11] will be IsTransparent

and param.l[3] will be 0.  If windowId is a normal opaque window,

param.b[11] will be IsOpaque and param.l[3] will contain the window's

icon window, if one has been defined with an X_SetIconWindow request.

If windowId is an icon window, param.b[11] will be IsIcon and

param.l[3] will contain the icon's corresponding "regular" window.

Errors: BadWindow

X_StoreName

code		21

windowId	Window

param.s[0]	length of name in characters (>= 0)

Assigns a name to a window.  This request must be followed by the

characters of the window name, followed by 0 t0 3 pad bytes to make

the length a multiple of four.

The name is typically used by a window manager (to create named icons,

for example).

Errors: BadWindow, BadValue

X_FetchName

code		22

windowId	Window

Returns the name of a window.

The reply:

param.s[0]	number of characters

The reply is followed by the specified number of characters of name,

followed by 0 to 3 pad bytes to make the length a multiple of four.

Errors: BadWindow

X_SetIconWindow

code		23

windowId	Window

param.l[0]	icon Window or 0

Sets/clears the icon window for a window.  The icon window must be a

sibling of the specified window, both windows must be opaque, and

neither can already be an icon window.

An icon window should be used when the client wants to control the

contents of the icon form.  A window manager should create an icon

window automatically if the client doesn't provide one.

Errors: BadWindow, BadMatch

X_SetResizeHint

code		24

windowId	Window

param.s[0]	base height (>= 0)

param.s[1]	height increment (> 0)

param.s[2]	base width (>= 0)

param.s[3]	width increment (> 0)

Defines the shape of a window.  The inside height of the window should

be the base height plus some multiple of the height increment, and the

inside width of the window should be the base width plus some multiple

of the width increment.  These parameters are hints, in that

X_ChangeWindow and X_ConfigureWindow do not check conformance.

A newly created window has a base height and width of zero, and height

and width increments of one.

The base height and width must be non-negative, and the height and

width increments must be positive.

The resize hints are typically used by a window manager.

Errors: BadWindow, BadValue

X_GetResizeHint

code		25

windowId	Window

Returns the resize parameters.

The reply:

param.s[0]	minimum height

param.s[1]	height increment

param.s[2]	minimum width

param.s[3]	width increment

Errors: BadWindow

X_DefineCursor

code		26

windowId	Window

param.l[0]	Cursor or 0

If a cursor is specified, it will be used when the mouse is in the

window.  If no cursor is specified, the parent's cursor will be used

when the mouse is in the window.

On the root window, with no cursor specified, the default cursor is

restored.

Errors: BadWindow, BadCursor

X_SelectInput

code		27

windowId	Window

param.l[0]	event mask

Defines which input events the window is interested in.  If a window is

not interested in an event, it usually will propagate up to the closest

ancestor that is interested.  The bits of the mask are:

KeyPressed	 0x0001		keyboard key pressed

KeyReleased	 0x0002		keyboard key released

ButtonPressed	 0x0004		mouse button pressed

ButtonReleased	 0x0008		mouse button released

EnterWindow	 0x0010		mouse entering window

LeaveWindow	 0x0020		mouse leaving window

MouseMoved	 0x0040		mouse moves within window

ExposeWindow	 0x0080		full window changed and/or exposed

ExposeRegion	 0x0100		region of window exposed

ExposeCopy	 0x0200		region exposed by X_CopyArea

RightDownMotion	 0x0400		mouse moves with right button down

MiddleDownMotion 0x0800		mouse moves with middle button down

LeftDownMotion	 0x1000		mouse moves with left button down

UnmapWindow	 0x2000		window is unmapped

FocusChange	 0x4000		keyboard focus changed

Selecting ExposeRegion also selects ExposeWindow.

Details of each kind of event are given later.

Overrides all previous selections on the same window by all clients.

If a window has both ButtonPressed and ButtonReleased selected, then a

ButtonPressed event in that window will automatically "grab" the mouse

until all buttons are released, with events sent to windows as

described for X_GrabMouse.

Input selection on the root window should be reserved for a window

manager.

Errors: BadWindow

X_GrabMouse

code		28

windowId	Window

param.l[0]	Cursor

param.l[1]	event mask

All future mouse events will only go to windows for which the grabbing

client has issued X_SelectInput commands.  (The event mask temporarily

overrides any X_SelectInput on the window.)  If the client has not

issued an X_SelectInput command on the window where the event would

normally be sent, then the event will be sent to the specified window,

provided the event is specified in the mask and is not EnterWindow or

LeaveWindow.  Only the bits ButtonPressed, ButtonReleased, EnterWindow,

LeaveWindow, MouseMoved, LeftDownMotion, MiddleDownMotion, and

RightDownMotion are useful in the mask.  The specified cursor will be

used regardless of what window the mouse is in.

This request will fail if someone else has already grabbed the mouse

and has not released it; the request overrides any other grab in

progress for this client.

If the request is successful there is a reply, but it contains no

information.

Errors: BadWindow, BadCursor, BadGrab

X_GrabButton

code		29

mask		button mask

windowId	Window

param.l[0]	Cursor

param.l[1]	event mask

The button mask must have exactly one of LeftMask, MiddleMask, or

RightMask set, and may have some combination of ControlMask, MetaMask,

ShiftMask, and ShiftLockMask set.  If the specified button is pressed

when exactly the specified keys are down, this and all future mouse

events are grabbed until all buttons are released, with events sent to

windows as described for X_GrabMouse.  During the grab, the specified

cursor will be used regardless of what window the mouse is in.

This request will fail if some other client has already grabbed the

same button/key combination and has not released it.

If the request is successful there is a reply, but it contains no

information.

Errors: BadWindow, BadCursor, BadValue, BadGrab

X_QueryMouse

code		30

windowId	Window

Returns the current mouse coordinates and the state of various keys

and buttons.

The reply:

param.l[0]	Window

param.s[2]	x coord

param.s[3]	y coord

param.s[4]	key and button state

The coordinates of the mouse relative to window are given, even if the

mouse is outside the window.  If the mouse is also in a mapped child

window, the child is returned, otherwise the return window is 0.

The high bits of the state parameter are the same as for the event

detail in keyboard and mouse events, and are defined further below

in the discussion of input events.

Errors: BadWindow

X_InterpretLocator

code		31

windowId	Window

param.l[0]	Locator

Interprets the coordinate with respect to the window.

The reply:

param.l[0]	Window

param.s[2]	x coord

param.s[3]	y coord

The coordinates of the locator relative to the window are given, even

if the locator is outside the window.  If the locator is also in a

mapped child window, the child is returned, otherwise the return window

is 0.

Errors: BadWindow

X_WarpMouse

code		32

windowId	destination Window

param.s[0]	destination x coord

param.s[1]	destination y coord

param.l[1]	source Window

param.s[4]	source height

param.s[5]	source width

param.s[6]	source left coord

param.s[7]	source top coord

Move the mouse to the destination position relative to the origin of

the destination window, but only if the mouse is currently in a visible

portion of the specified region of the source window.

If the source height is zero, the current height of the source window

minus the source top coord is used.  If the source width is zero, the

current width of the source window minus the source left coord is used.

Errors: BadWindow

X_FocusKeyboard

code		33

func		0 (for future expansion)

windowId	Window

The specified window becomes the keyboard "focus" window.  If the

window that would normally receive a KeyPressed or KeyReleased event is

not (a descendant of) the focus window, then the event will be directed

at the focus window instead.

Generates a FocusChange event with detail LeaveWindow on the current

focus window, and a FocusChange event with detail EnterWindow on the

new focus window.

The default focus window is the root window.  If the focus window is

destroyed, its parent window becomes the focus window.

Errors: BadWindow

X_CircWindowDown

code		34

windowId	Window

Lower the highest mapped child of this window that (partially) obscures

another child.  Repeated executions lead to round robin lowering.

Generates exposure events on formerly obscured opaque windows.

Errors: BadWindow

X_QueryTree

code		35

windowId	Window

Returns the parent and child windows of the specified window.

The reply:

param.l[0]	parent Window (or 0 if none)

param.l[1]	number of child windows

The reply is followed by the specified number of child Window ids, each

Window id being four bytes long.  The children are listed in current

stacking order, from bottom-most (first) to top-most (last).

Errors: BadWindow

X_Clear

code		40

windowId	Window

Clear the window and repaint it with the background.  The tiling origin

is controlled by the tilemode.

A transparent window inherits its parent's background for this

operation.

Errors: BadWindow

X_PixFill

code		41

func		display function (0-15)

mask		plane mask

windowId	Window

param.s[0]	destination height

param.s[1]	destination width

param.s[2]	destination left coord

param.s[3]	destination top coord

param.u[4]	source pixel

param.l[3]	mask Bitmap or 0

Performs a function in a region of the window.  The source pixel

defines the value of the source bit for each plane, and the plane mask

defines which destination bit planes are affected.  The display

function is computed on each bit plane.  If no mask bitmap is

specified, the entire destination is affected.  If a mask bitmap is

specified, it defines the shape of the source and which pixels of the

destination will be affected.

Errors: BadWindow, BadValue, BadBitmap

X_TileFill

code		42

func		display function (0-15)

mask		plane mask

windowId	Window

param.s[0]	destination height

param.s[1]	destination width

param.s[2]	destination left coord

param.s[3]	destination top coord

param.l[2]	tile Pixmap

param.l[3]	mask Bitmap or 0

Performs a function in a region of the window using a repeating pattern

defined by the tile pixmap.  The tiling origin is controlled by the

window's tilemode.  The plane mask defines which destination bit planes

are affected.  The display function is computed on each bit plane.  If

no mask bitmap is specified, the entire destination is affected.  If a

mask bitmap is specified, it defines which pixels of the destination

will be affected, and must be the same height and width as the

destination.

Errors: BadWindow, BadValue, BadPixmap, BadTile, BadBitmap, BadMatch

X_PixmapPut

code		43

func		display function (0-15)

mask		plane mask

windowId	Window

param.s[0]	source height

param.s[1]	source width

param.s[2]	source left coord

param.s[3]	source top coord

param.l[2]	source Pixmap

param.s[6]	destination left coord

param.s[7]	destination top coord

Performs a function in a region of the window using a region of a

pixmap.  The source height, width, and coordinates specify the region

of the source pixmap to be used.  The plane mask defines which

destination bit planes are affected.  The display function is computed

on each bit plane.

Errors: BadWindow, BadValue, BadPixmap

X_PixmapBitsPut

code		44

func		display function (0-15)

mask		plane mask

windowId	Window

param.s[0]	source height (> 0)

param.s[1]	source width (> 0)

param.s[2]	destination left coord

param.s[3]	destination top coord

param.s[4]	0: XYFormat

1: ZFormat

param.l[3]	mask Bitmap or 0

Performs a function in a region of the window using a pixmap.  The

plane mask defines which destination bit planes are affected.  The

display function is computed on each bit plane.  If a mask bitmap is

specified, it defines which pixels of the destination will be affected,

and must be the same height and width as the source.

The request must be followed by the data bytes of the source pixmap in

the specified format, followed by 0 to 3 pad bytes to make the length

a multiple of four.

Errors: BadWindow, BadValue, BadBitmap, BadMatch

X_BitmapBitsPut

code		45

func		display function (0-15)

mask		plane mask

windowId	Window

param.s[0]	source height (> 0)

param.s[1]	source width (> 0)

param.s[2]	destination left coord

param.s[3]	destination top coord

param.u[4]	foreground pixel

param.u[5]	background pixel

param.l[3]	mask Bitmap or 0

Performs a function in a region of the window using a pixmap defined by

a bitmap and a pair of source pixels.  The foreground pixel defines the

source for the one bits in the bitmap, and the background pixel defines

the source for the zero bits.  The plane mask defines which destination

bit planes are affected.  The display function is computed on each bit

plane.  If a mask bitmap is specified, it defines which pixels of the

destination will be affected, and must be the same height and width as

the source.

The request must be followed by the data bytes of the source bitmap in

bitmap format, followed by 0 or 2 pad bytes to make the length a

multiple of four.

Errors: BadWindow, BadValue, BadBitmap, BadMatch

X_CopyArea

code		46

func		display function (0-15)

mask		plane mask

windowId	Window

param.s[0]	source height

param.s[1]	source width

param.s[2]	source left coord

param.s[3]	source top coord

param.s[6]	destination left coord

param.s[7]	destination top coord

Copies one region of the window to another region in the same window.

The plane mask defines which destination bit planes are affected.  The

display function is computed on each bit plane.

If parts of the source are obscured, the corresponding parts of the

destination are filled with the window's background.  If ExposeCopy has

been selected, ExposeRegion events will be generated for those parts of

the destination, and then an ExposeCopy event will be generated.  All

of these events are guaranteed to be together in the stream, with no

intervening events.

Errors: BadWindow, BadValue

X_Text

code		47

func		display function (0-15)

mask		plane mask

windowId	Window

param.s[0]	destination left coord

param.s[1]	destination top coord

param.l[1]	Font

param.u[4]	foreground pixel

param.u[5]	background pixel

param.s[6]	number of characters (>= 0)

param.b[14]	inter-character pad

param.b[15]	space character pad

Draws text using the specified function.  The coordinates are for the

upper left of the first character.  The foreground pixel defines the

source for the one bits in the font character bitmaps, and the

background pixel defines the source for the zero bits.  The plane mask

defines which destination bit planes are affected.  The display

function is computed on each bit plane.  The inter-character pad

specifies the number of pixels to skip after each character before

printing the next character.  The space character pad specifies the

number of additional pixels to skip after each space character before

printing the next character.  The skipped pixels are not considered

part of the source or destination, and are not altered.

The request must be followed by the specified number of characters,

followed by 0 to 3 pad bytes to make the length a multiple of four.

Errors: BadWindow, BadValue, BadFont

X_TextMask

code		48

func		display function (0-15)

mask		plane mask

windowId	Window

param.s[0]	destination left coord

param.s[1]	destination top coord

param.l[1]	Font

param.u[4]	source pixel

param.s[6]	number of characters (>= 0)

param.b[14]	inter-character pad

param.b[15]	space character pad

Like X_Text, but the source pixel defines the value of the source bit

for each plane, and the font character bitmaps are used as masks to

define which pixels of the destination are affected.

The request must be followed by the specified number of characters,

followed by 0 to 3 pad bytes to make the length a multiple of four.

Errors: BadWindow, BadValue, BadFont

X_Line

code		49

func		display function (0-15)

mask		plane mask

windowId	Window

param.s[0]	x1 coord

param.s[1]	y1 coord

param.s[2]	x2 coord

param.s[3]	y2 coord

param.u[4]	source pixel

param.b[10]	brush height (> 0)

param.b[11]	brush width (> 0)

This request is the same as a DrawSolidLine X_Draw request with the two

vertexes (x1, y1, VertexDontDraw) and (x2, y2, VertexDrawLastPoint).

Errors: BadWindow, BadValue

X_Draw

code		50

func		display function (0-15)

mask		plane mask

windowId	Window

param.s[0]	number of vertexes (>= 0)

param.u[1]	source pixel

param.b[4]	brush height (> 0)

param.b[5]	brush width (> 0)

param.s[3]	0: DrawSolidLine

1: DrawDashedLine

2: DrawPatternedLine

param.u[4]	alternate source pixel (for patterned line)

param.s[5]	pattern string (for dashed or patterned line)

param.s[6]	pattern length (for dashed or patterned line) (1-16)

param.s[7]	pattern multiplier (for dashed or patterned line) (> 0)

Draws arbitrary polygons/curves using the specified function and brush

rectangle.  The area covered should be that obtained by laying down the

brush rectangle at every point along the path, with the upper left

corner following the path.  Each pixel in that area may be processed

just once, or the brush may be repainted for each point along the path.

The plane mask defines which destination bit planes are affected.  The

display function is computed on each bit plane.  For a solid line, the

source pixel defines the value of the source bit for each plane.  For

a dashed or patterned line, the pattern string specifies up to 16 bits

of pattern; the pattern length specifies the number of bits.  The

pattern multiplier specifies the number of times each bit in the string

should be repeated before moving to the next bit.  The bits are used

least significant first, wrapping as needed.  For a dashed line, the

destination is only updated when the pattern bit is 1.  For a patterned

line, the alternate source pixel is used when the pattern bit is 0.

The request must be followed by the specified number of vertexes,

followed by 0 or 2 pad bytes to make the length a multiple of four.

The C definition of a vertex is:

typedef struct _Vertex {

short x, y;

unsigned short flags;

} Vertex;

The flags are as follows:

VertexRelative		0x0001		else absolute

VertexDontDraw		0x0002		else draw

VertexCurved		0x0004		else straight

VertexStartClosed	0x0008		else not

VertexEndClosed		0x0010		else not

VertexDrawLastPoint	0x0020		else don't

A relative vertex is expressed in terms of offsets from the previous

vertex, an absolute vertex has offsets from the origin of the window.

The first vertex is never relative.  VertexDontDraw and VertexCurved

control drawing from the previous vertex to the current vertex.  It

can be useful to combine VertexDontDraw and VertexCurved to define the

shape of the displayed portion of the curve.  VertexDontDraw can also

be used to combine multiple draws in one request.  VertexStartClosed

should be set in the first vertex of a closed curve, VertexEndClosed in

the last; the two should specify the same point.  In drawing from the

previous vertex to the current vertex, the current vertex point is not

drawn unless VertexDrawLastPoint is set.

Errors: BadWindow, BadValue

X_DrawFilled

code		51

func		display function (0-15)

mask		plane mask

windowId	Window

param.s[0]	number of vertexes (>= 0)

param.u[1]	source pixel

param.l[1]	tile Pixmap or 0

Draws arbitrary filled polygons/curves using the specified function

tiling pixmap or source pixel.  If a tile is specified, the tiling

origin is controlled by the window's tilemode.  If no tile is given,

the source pixel is used instead.  The plane mask defines which

destination planes are affected.

The request must be followed by the specified number of vertexes,

followed by 0 or 2 pad bytes to make the length a multiple of four.

The vertex list should only consist of one or more closed regions.

A point is defined to be inside a region if an infinite ray with the

point as an origin crosses the path of the region an odd number of

times.

Errors: BadWindow, BadValue, BadPixmap, BadTile

X_PixmapSave

code		52

windowId	Window

param.s[0]	height (> 0)

param.s[1]	width (> 0)

param.s[2]	left coord

param.s[3]	top coord

Creates a pixmap from the given portion of the window.  The pixmap will

contain a direct image of that portion of the screen, including any

visible portions of subwindows or overlapping windows.

The window must be mapped, and it must be the case that, if there were

no subwindows or overlapping windows, the specified portion of the

window would be fully visible on the screen.

The reply:

param.l[0]	Pixmap

Errors: BadWindow, BadValue, BadAlloc

X_PixmapGet

code		53

func		0: XYFormat

1: ZFormat

windowId	Window

param.s[0]	height (> 0)

param.s[1]	width (> 0)

param.s[2]	left coord

param.s[3]	top coord

Returns the contents of the given portion of the window in the given

pixmap format.  The pixmap will contain a direct image of that portion

of the screen, including any visible portions of subwindows or

overlapping windows.

The window must be mapped, and it must be the case that, if there were

no subwindows or overlapping windows, the specified portion of the

window would be fully visible on the screen.

The reply:

param.l[0]	number of bytes

The reply is followed by the specified number of bytes of pixmap data,

followed by 0 to 3 pad bytes to make the length a multiple of four.

Errors: BadWindow, BadValue

X_SetUp

code		80

Returns the root window and device information.

The reply:

param.l[0]	root Window

param.s[2]	protocol version number (10)

param.s[3]	device identifier

param.s[4]	number of bit planes

param.u[5]	number of usable color map cells

The currently defined devices are:

XDEV_VS100	1

XDEV_QVSS	2

XDEV_QDSS	3

X_UngrabMouse

code		81

Releases hold of the mouse if it was grabbed via an X_GrabMouse.

X_UngrabButton

code		82

mask		button mask

Releases hold of the button/key combination if it was grabbed.  The

button mask must have exactly one of LeftMask, MiddleMask, or RightMask

set, and may have some combination of ControlMask, MetaMask, ShiftMask,

and ShiftLockMask set.

Errors: BadValue

X_GetColor

code		83

param.u[0]	red value

param.u[1]	green value

param.u[2]	blue value

Determines the closest color provided by the hardware, and returns a

pixel value representing that color.  The corresponding color map cell

is read-only.

Read-only color map cells are shared among clients, so this request may

simply reference count an existing cell.

The reply:

param.u[0]	pixel

Errors: BadAlloc

X_GetColorCells

code		84

func		1 if planes must be contiguous, 0 otherwise

param.s[0]	number of colors (>= 0)

param.s[1]	number of planes (>= 0)

Allocates N * 2^P color map cells, where N is the number of colors and

P is the number of planes specified.

The reply:

param.u[0]	plane mask

The bits in the plane mask will be contiguous if requested.  The reply

is followed by 2 * N bytes of data, where N is the number of colors

specified, followed by 0 or 2 pad bytes to make the length a multiple

of four.  Each 16-bit word represents the pixel value of one of the

color cells.  Additional pixel values are obtained by oring in all

possible combinations of one bits from the plane mask.  The initial

colors for all of these cells is undefined.

If zero colors are requested, then the request allocates all cells with

a pixel value having at least one non-zero bit in the plane mask.  At

most one such request will succeed.

Errors: BadValue, BadAlloc

X_FreeColors

code		85

mask		plane mask

param.s[0]	number of colors (>= 0)

Frees several colors or color map cells.  Further use of the given

pixel values will result in undefined colors.

The request must be followed by the specified number of 16-bit pixel

values, followed by 0 or 2 pad bytes to make the length a multiple of

four.  Additional pixel values are obtained by oring in all possible

combinations of one bits from the plane mask.

Errors: BadValue, BadAccess

X_StoreColors

code		86

param.s[0]	number of colors (>= 0)

Change the colors of several pixels to the closest available hardware

colors.

The request must be followed by the specified number of color

definitions.  The C definition is:

typedef struct _ColorDef {

unsigned short pixel;

unsigned short red, green, blue;

} ColorDef;

Errors: BadValue, BadAccess

X_QueryColor

code		87

param.u[0]	pixel

Returns the color values for a pixel.

The reply:

param.u[0]	red value

param.u[1]	green value

param.u[2]	blue value

Errors: BadValue

X_GetFont

code		88

param.s[0]	length of name in chars (> 0)

Loads a font.  This request must be followed by the characters of the

font name, followed by 0 to 3 pad bytes to make the length a multiple

of four.  Case is significant.

Fonts are shared among clients, so this request may simply reference

count an existing font.

The reply:

param.l[0]	Font

Errors: BadValue, BadFont, BadAlloc

X_FreeFont

code		89

param.l[0]	Font

Indicates that the font is no longer needed.  The font should never

again be referenced.

Errors: BadFont

X_QueryFont

code		90

param.l[0]	Font

Returns information about a font.

The reply:

param.s[0]	height

param.s[1]	"average" width

param.s[2]	first character

param.s[3]	last character

param.s[4]	baseline

param.s[5]	1 if fixed width, 0 if variable width

The baseline specifies where in pixels from the bottom of the font the

characters without descenders begin.

A font is fixed width if all characters in the given range are the same

width.

Fonts will get more complicated in a future version.

Errors: BadFont

X_CharWidths

code		91

param.l[0]	Font

param.s[2]	number of characters (>= 0)

Returns the width in pixels of each character.  The request must be

followed by the specified number of characters, followed by 0 to 3 pad

bytes to make the length a multiple of four.

The reply:

param.l[0]	number of bytes

The reply is followed by the specified number of bytes of data,

followed by 0 or 2 pad bytes to make the length a multiple of four.

Each 16-bit word of data contains the width of a character.

Errors: BadFont, BadValue

X_StringWidth

code		92

param.l[0]	Font

param.s[2]	number of characters (>= 0)

Returns the width in pixels of a string in a font.

The request must be followed by the specified number of characters,

followed by 0 to 3 pad bytes to make the length a multiple of four.

The reply:

param.s[0]	width in pixels

Errors: BadFont, BadValue

X_FontWidths

code		93

param.l[0]	Font

Returns the widths in pixels of all characters in a font.

The reply:

param.l[0]	number of bytes

The reply is followed by the specified number of bytes of data,

followed by 0 or 2 pad bytes to make the length a multiple of four.

Each 16-bit word of data contains the width of a character.  The widths

are for the range of characters given by X_QueryFont.

Errors: BadFont

X_StoreBitmap

code		94

param.s[0]	height (> 0)

param.s[1]	width (> 0)

Creates a bitmap.

The request must be followed by the correct number of bytes of data in

bitmap format, followed by 0 or 2 pad bytes to make the length a

multiple of four.

The reply:

param.l[0]	Bitmap

Errors: BadValue, BadAlloc

X_FreeBitmap

code		95

param.l[0]	Bitmap

Frees the storage consumed by the bitmap.  The bitmap should never be

referenced again.

Errors: BadBitmap

X_CharBitmap

code		96

param.l[0]	Font

param.s[2]	character

Copies a character bitmap from a font.

The font can be freed immediately if no further explicit references to

it are to be made.

The reply:

param.l[0]	Bitmap

Errors: BadFont, BadValue, BadAlloc

X_StorePixmap

code		97

func		0: XYFormat

1: ZFormat

param.s[0]	height (> 0)

param.s[1]	width (> 0)

Creates a pixmap.

The request must be followed by the correct number of bytes of data the

specified format, followed by 0 to 3 pad bytes to make the length a

multiple of four.

The reply:

param.l[0]	Pixmap

Errors: BadValue, BadAlloc

X_FreePixmap

code		98

param.l[0]	Pixmap

Frees the storage consumed by the pixmap.  The pixmap should never be

referenced again.

Errors: BadPixmap

X_MakePixmap

code		99

param.l[0]	Bitmap or 0

param.u[2]	foreground pixel

param.u[3]	background pixel

Creates a pixmap from a bitmap.  The foreground pixel is used for the

one bits in the bitmap, and the background pixel is used for the zero

bits.  If no bitmap is given, a bitmap of all one bits suitable for use

as a tiling pixmap is used.

The bitmap can be freed immediately if no further explicit references

to it are to be made.

The reply:

param.l[0]	Pixmap

Errors: BadBitmap, BadValue, BadAlloc

X_QueryShape

code		100

func		0: CursorShape

1: TileShape

2: BrushShape

param.s[0]	height (> 0)

param.s[1]	width (> 0)

Given a rectangular shape, returns the "closest" shape actually

supported by the display for a given purpose.  For a cursor shape,

returns a Bitmap shape acceptable for X_StoreCursor.  For a tile shape,

returns a Pixmap shape acceptable for tiling.  For a brush shape,

returns a shape acceptable for X_Line and X_Draw.

The reply:

param.s[0]	height

param.s[1]	width

Errors: BadValue

X_StoreCursor

code		101

func		display function (0-15)

param.l[0]	cursor Bitmap

param.u[2]	foreground pixel

param.u[3]	background pixel

param.l[2]	mask Bitmap or 0

param.s[6]	x offset

param.s[7]	y offset

Defines a mouse cursor.  The foreground pixel is used for the one bits

in the cursor bitmap, and the background pixel is used for the zero

bits.  The mask bitmap defines the shape of the cursor; that is, the

one bits in the mask define which cursor pixels will be displayed.  If

no mask is given, all pixels of the cursor are displayed.  The mask

bitmap, if present, must be the same size as the cursor bitmap.  The

offsets define the point that actually corresponds to the mouse

position; this must be a point in the cursor bitmap.

The components of the cursor may be transformed arbitrarily to meet

hardware limitations.

The bitmaps can be freed immediately if no further explicit references

to them are to be made.

The reply:

param.l[0]	Cursor

Errors: BadValue, BadBitmap, BadMatch, BadAlloc

X_FreeCursor

code		102

param.l[0]	Cursor

Frees the storage consumed by the cursor.  The cursor should never be

referenced again.

Errors: BadCursor

X_MouseControl

code		103

param.s[0]	acceleration (>= 1)

param.s[1]	threshold (>= 0)

Defines how the mouse moves.  The acceleration is a multiplier for

movement.  E.g., specifying 3 means the cursor moves three times as

fast as the mouse.  Acceleration only takes effect if the mouse moves

more than threshold pixels at once, and only applies to the amount

beyond the threshold.

Errors: BadValue

X_FeepControl

code		104

func		volume (0-7)

Defines the base volume for X_Feep requests.  The volume is in the

range 0 to 7, with 7 the loudest.

Errors: BadValue

X_Feep

code		105

param.s[0]	relative volume (-7 to 7)

Cause an audible bell.  The volume is added to the base volume defined

by the X_FeepControl request, the sum limited to the range 0 to 7.

Errors: BadValue

X_ShiftLock

code		106

func		0: LockUpDownMode

1: LockToggleMode

Sets the mode of the Shift LOCK key on the keyboard.  When the keyboard

is in LockUpDownMode, KeyPressed and KeyReleased events are sent as for

any other key, and the ShiftLockMask sent in events gives the current

state of the key.  In LockToggleMode, KeyPressed and KeyReleased events

are never sent for the LOCK key, and the state of the ShiftLockMask

sent in events is toggled on every press of the LOCK key.

The key is initially in LockToggleMode.

Errors: BadValue

X_KeyClick

code		107

func		volume (0-8)

Turns keyboard key click off (volume 0), or turns it on and sets the

volume, with 8 the loudest.

Errors: BadValue

X_AutoRepeat

code		108

func		0 for off, 1 for on

Turns keyboard autorepeat on or off.

Errors: BadValue

X_ScreenSaver

code		109

func		0 for video off, 1 for video on

param.s[0]	screen saver timeout in minutes (> 0)

param.s[1]	pattern change timeout interval in minutes (> 0)

If the server remains idle for the specified number of minutes, screen

saver is enabled.  If video off is specified, and the hardware supports

video blanking, the screen will simply go blank.  Otherwise, the

screen will be tiled with the root window background tile, randomly

re-origined at the specified timeout interval.  The screen state will

be restored when the next request or input event occurs.

Errors: BadValue

X_StoreBytes

code		110

func		cut buffer (0-7)

param.s[0]	number of bytes (>= 0)

Stores an arbitrary string of bytes one of eight "cut buffers".  These

bytes may be retrieved with the X_FetchBytes request.

The previous contents of the cut buffer are lost.

The request must be followed by the specified number of bytes of data,

followed by 0 to 3 pad bytes to make the length a multiple of four.

Errors: BadValue

X_FetchBytes

code		111

func		cut buffer (0-7)

Retrieves the contents of the specified cut buffer.

The reply:

param.s[0]	number of bytes

The reply is followed by the specified number of bytes of data,

followed by 0 to 3 pad bytes to make the length a multiple of four.

Errors: BadValue

X_RotateCuts

code		112

func		rotate count (0-7)

Rotates the cut buffers by N.  Buffer 0 becomes buffer N, buffer 1

becomes N+1 mod 8, and so on.

Errors: BadValue

X_AddHost

code		113

func		address family

param.l[0-?]	host address

Add the specified host to the list of hosts allowed to open

connections.

The currently recognized address families are

XAF_INET		2

XAF_DECnet		12

In Unix term, the host address for XAF_INET is "struct in_addr",

and for XAF_DECnet is "struct dn_naddr".

The client must reside on the same host as the window system.

Under Unix, the initial set of hosts consists of the host the window

system is running on, plus those hosts listed in /etc/X?.hosts, where ?

is the number of the display.  This file should consist of host names

separated by newlines.  DECnet host names must have a trailing "::".

Errors: BadValue, BadAccess

X_RemoveHost

code		114

func		address family

param.l[0-?]	host address

Remove the specified host from the list of hosts allowed to open

connections.

The address family and host address are as for X_AddHost.

The client must reside on the same host as the window system.

Errors: BadValue, BadAccess

X_GetHosts

code		115

func		address family

Returns the current list of hosts allowed to open connections.

The reply:

param.l[0]	number of bytes

The reply is followed by the specified number of bytes of data,

containing a list of host addresses.  The size of each host address

is determined by the address family, as for X_AddHost.

Errors: BadValue

X_GrabServer

code		116

Disables processing of requests and close-downs on all other

connections (than the one this request arrived on).

X_UngrabServer

code		117

Restarts processing of requests and close-downs on other connections.

X_LookupColor

code		118

param.s[0]	length of name in characters (>= 0)

Returns the color values for a given color name.  The name is looked

up in a standard color database.  This request must be followed by the

characters of the color name, followed by 0 to 3 pad bytes to make the

length a multiple of four.  Case is significant.

The reply:

param.u[0]	exact red value

param.u[1]	exact green value

param.u[2]	exact blue value

param.u[3]	closest hardware red value

param.u[4]	closest hardware green value

param.u[5]	closest hardware blue value

Errors: BadValue, BadColor

Input events:

Selecting MouseMoved will result in events independent of the state of the

mouse buttons.  By selecting some subset of (LeftDownMotion, MiddleDownMotion,

RightDownMotion) instead, MouseMoved events will only be generated when one or

more of the specified buttons is depressed.

KeyPressed, KeyReleased, ButtonPressed, ButtonReleased, and MouseMoved events

are usually sent to the smallest window enclosing the mouse that has selected

such events.  For KeyPressed and KeyReleased events, if this window is not in

the keyboard focus hierarchy, the events will be sent to the focus window

instead.

KeyPressed, KeyReleased, ButtonPressed, ButtonReleased, EnterWindow,

LeaveWindow, and MouseMoved events have the following structure:

code		kind of event (KeyPressed, etc.)

param.l[0]	event Window

param.s[2]	time in 10 millisecond ticks (Key and Button only)

param.s[3]	event detail

param.s[4]	mouse x coord within event window

param.s[5]	mouse y coord within event window

param.l[3]	sub Window

param.l[4]	Locator

The coordinates of the mouse relative to the event window are reported, even if

the mouse is not in the window (because of grabbing or keyboard focusing).  If

the mouse is also in a (direct) child of the event window, the subwindow is set

to that child, otherwise the subwindow is 0.  The locator defines the mouse

coordinates in absolute terms.

The time value is present only for KeyPressed, KeyReleased, ButtonPressed, and

ButtonReleased events.  Note that there are only 16 bits of time (which should

be treated as unsigned), which wraps after approximately 11 minutes, so only

time differences between clustered events is interesting.

For all seven event types, the high bits of the detail encode the state of

various keys and buttons just before the event:

ControlMask	0x4000			Control key

MetaMask	0x2000			Meta (Symbol) key

ShiftMask	0x1000			Shift key

ShiftLockMask	0x0800			ShiftLock key

LeftMask	0x0400			Left button

MiddleMask	0x0200			Middle button

RightMask	0x0100			Right button

For KeyPressed and KeyReleased, the low byte of the detail gives the key.  This

is not an ASCII character, but the code transmitted by the LK201 keyboard.

For ButtonPressed and ButtonReleased, the low byte of the detail is one of:

RightButton	0

MiddleButton	1

LeftButton	2

For EnterWindow and LeaveWindow, the low byte of the detail is either zero or

one of:

IntoOrFromSubwindow	1

VirtualCrossing		2

EnterWindow and LeaveWindow events are generated as follows:

When the mouse moves from window A to window B, and B is an ancestor of A:

A will get a LeaveWindow with detail 0

windows between A and B exclusive that have LeaveWindow selected

will get a LeaveWindow with detail 2

B will get an EnterWindow with detail 1

When the mouse moves from window A to window B, and B is a descendant of A:

A will get a LeaveWindow with detail 1

windows between A and B exclusive that have EnterWindow selected

will get an EnterWindow with detail 2

B will get an EnterWindow with detail 0

When the mouse moves from window A to window B, with window C being their

least common ancestor:

A will get a LeaveWindow with detail 0

windows between A and C exclusive that have LeaveWindow selected

will get a LeaveWindow with detail 2

windows between C and B exclusive that have EnterWindow selected

will get an EnterWindow with detail 2

B will get an EnterWindow with detail 0

At the start of a mouse grab, either automatically from a button press,

or from an X_GrabMouse or X_GrabButton, with the mouse in window A, and

with window B being the smallest window enclosing the mouse that has had

an X_SelectInput issued on it by some client:

A will get a LeaveWindow with detail 0 if the grabbing client has

not issued an X_SelectInput command on B

ancestors of A (not including the root) will get a LeaveWindow with

detail 2 if the grabbing client has not issued an X_SelectInput

on the window and the window has LeaveWindow selected.

At the end of a mouse grab, with the mouse in window A, and with window B

being the smallest window enclosing the mouse that has had an X_SelectInput

issued on it by some client:

ancestors of A (not including the root) will get an EnterWindow with

detail 2 if the grabbing client has not issued an X_SelectInput

on the window and the window has EnterWindow selected.

A will get an EnterWindow with detail 0 if the grabbing client has

not issued an X_SelectInput command on B.

Note that EnterWindow and LeaveWindow events with detail 0 or 1 (but not 2)

will propagate to the smallest enclosing window that has actually selected the

event.

LeaveWindow events are not generated when windows are unmapped or destroyed.

UnmapWindow events occur whenever an X_UnmapWindow or X_UnmapSubwindows request

is executed on a mapped window.  The event structure is:

code		UnmapWindow

param.l[0]	event Window

param.l[3]	sub Window

If a subwindow is given, it is the actual window on which the request was

issued (not the ancestor that is a direct child of the event window).

FocusChange events occur whenever the keyboard focus changes.  The event

structure is:

code		FocusChange

param.l[0]	event Window

param.s[3]	EnterWindow or LeaveWindow

param.l[3]	sub Window

If a subwindow is given, it is the actual window on which the request was

issued (not the ancestor that is a direct child of the event window).

For ExposeWindow and ExposeRegion events, the structure is as follows:

code		ExposeWindow or ExposeRegion

param.l[0]	event Window

param.s[3]	detail (0 or ExposeCopy)

param.s[4]	width of area

param.s[5]	height of area

param.l[3]	sub Window

param.s[8]	top coord of area

param.s[9]	left coord of area

Coordinates are relative to the inside of the exposed window.

ExposeWindow and ExposeRegion events are triggered as (parts of) windows become

exposed.  When an entire window becomes exposed (as when a window is mapped or

changes size), an ExposeWindow event is sent.  The width and height of the

entire window is given, and the coordinates are (0, 0).  When only parts of a

window become exposed (as when an obscuring window is moved), ExposeRegion

events are sent describing each newly exposed area.  However, if only

ExposeWindow has been selected, a single ExposeWindow will be sent instead.  If

the region exposure is the result of a CopyArea, then ExposeCopy will be set in

the detail word.  If the exposure is actually that of a descendant of the

window selecting the event, the subwindow is set to that descendant and the

coordinates are actually for the subwindow, otherwise the subwindow is 0.  For

a given window exposure or CopyArea, all resulting ExposeRegion events will be

sent contiguously, with no other events interspersed.

For ExposeCopy events, the structure is as follows:

code		ExposeCopy

param.l[0]	event Window

param.l[3]	sub Window

If the CopyArea was done in a descendant of the window selecting the event, the

subwindow is set to that descendant, otherwise the subwindow is 0.